Raspberry_Pi_4_RTL-SDR_Survey Per ChatGPT

Steve:

Develop a plan to monitor, record, and characterize traffic on local 70 cm frequencies using a Raspberry Pi 4 and RTL-SDR.

ChatGPT:

Here’s a practical, flexible plan to monitor, record, and characterize 70 cm band traffic using a Raspberry Pi 4 and an RTL-SDR USB dongle. The goal is to observe both analog FM and digital modes (like D-STAR), log frequency activity, and optionally capture audio or raw IQ data for deeper analysis.

Project: 70 cm Band Traffic Monitoring System

Platform: Raspberry Pi 4

Receiver: RTL-SDR (e.g., RTL2832U + R820T2)

Band: 70 cm (typically 440–450 MHz)

Modes: Analog FM, D-STAR (DV), others optionally

1. Hardware Setup

Required:

- Raspberry Pi 4 (with active cooling recommended)

- MicroSD card (16 GB or larger)

- RTL-SDR dongle

- USB extension cable (optional, but reduces RF noise)

- External antenna (UHF vertical preferred)

Optional:

- Active cooling (for long monitoring sessions)

- Wi-Fi or Ethernet internet access (for remote logging/updates)

2. Software Setup

Install OS and SDR Tools:

sudo apt update
sudo apt install rtl-sdr sox git python3-pip screen

Disable DVB driver (conflicts with RTL-SDR):

echo 'blacklist dvb_usb_rtl28xxu' | sudo tee /etc/modprobe.d/no-rtl.conf
sudo reboot

Useful Tools:

| Tool | Purpose |
| --- | --- |
| rtl_power | Frequency sweeping/logging |
| rtl_fm | FM audio capture |
| gqrx | GUI SDR (optional w/ VNC or HDMI) |
| SDRangel / CubicSDR | Optional GUI apps if using desktop |

You can also install inspectrum, baudline, or gnuradio if you want to analyze D-STAR or DV signals later.

3. Scanning & Logging Traffic

A. Logging activity using rtl_power

Scan the 440–450 MHz band and log signal strength (power) over time.

rtl_power -f 440M:450M:25k -i 15s -e 1h traffic.csv

- -f = frequency range and step size

- -i = integration interval

- -e = duration

- Output is a CSV you can graph later with Python, Excel, or heatmap.py from keenerd/rtl-sdr

B. Real-time monitoring with rtl_fm + SoX

To listen or record active frequencies:

rtl_fm -f 446.000M -M fm -s 22050 - | sox -t raw -r 22.05k -e signed -b 16 -c 1 - audio.wav

This saves audio from a specific simplex channel.

4. Characterizing Traffic

Logging & Analysis Suggestions:

- Use a Python script to parse rtl_power logs and identify active channels.

- Compare signal strength over time to determine most active frequencies.

- Optionally create heatmaps or histograms of activity.

- Record audio (for analog) or raw IQ (for digital) on active channels for later decoding.

Optional IQ capture:

Use rtl_sdr to record raw signal for later analysis:

rtl_sdr -f 445525000 -s 2048000 -n 1e6 output.iq

Then analyze with tools like:

- Inspectrum

- DSDPlus (for D-STAR, DMR, etc. — needs Wine or a Windows host)

- GNU Radio (for custom decoding chains)

5. Automation Plan

Crontab Job (example for hourly scan):

crontab -e

Add:

0 * * * * /home/pi/monitor/scan.sh >> /home/pi/logs/traffic.log 2>&1

Where scan.sh runs your rtl_power script and parses logs.

6. Optional Enhancements

- Web dashboard using Flask or Grafana + InfluxDB for real-time visualization

- Alert system (email/SMS) for new signals on rare frequencies

- Frequency hopping scan script for known simplex channels

Would you like:

- A sample scan.sh script?

- A Python script to graph and summarize rtl_power logs?

- A tutorial on capturing and decoding D-STAR DV with your setup?

Let me know what level of automation or analysis you want to build toward.

Steve:

First 2 Experiments

- Identify locally used and unused frequencies.

- Capture and decode D-Star DV.

ChatGPT:

Great! Here’s a clear outline for your first two experiments using the Raspberry Pi 4 and RTL-SDR:

Experiment 1: Identify Locally Used and Unused 70 cm Frequencies

Objective:

Detect which 70 cm simplex frequencies are active or quiet in your local area.

Tools:

- rtl_power (for frequency sweep)

- Python + CSV or heatmap.py (for visualization)

Step-by-Step Plan:

- Scan the Band:
Use rtl_power to log power levels across the simplex range:

rtl_power -f 440M:450M:25k -i 15s -e 12h bandlog.csv

  - 25k step size matches common channel spacing.

  - -i 15s averages readings every 15 seconds.

  - -e 12h or longer gives a meaningful baseline.

- Visualize Results:
Use keenerd’s heatmap.py to generate a heatmap:

python3 heatmap.py bandlog.csv output.png

- This will show where traffic occurs (bright spots) and where it’s quiet.

- Analyze:

  - Highlight active frequencies (e.g., 446.000 MHz).

  - Note unused ones that are clean enough for experiments or future simplex QSO tests.

Experiment 2: Capture and Decode D-STAR DV Traffic

Objective:

Record and decode D-STAR transmissions on simplex or repeater frequencies.

Tools:

- rtl_sdr or gqrx (to record IQ data)

- dsd or DSDPlus (to decode D-STAR DV)

- inspectrum or gnuradio (optional, for analysis)

Step-by-Step Plan:

- Tune to a D-STAR Frequency:
Use known D-STAR simplex or repeater frequencies, e.g.:

445.525 MHz (simplex)

- Record IQ Data:
Capture raw radio signal for a few minutes:

rtl_sdr -f 445525000 -s 2048000 -n 20480000 dstar.iq

- This records 10 seconds of IQ data at 2.048 Msps.

- Decode D-STAR with DSD:

  - Install dsd (Linux-native) or DSDPlus (Windows).

  - Play the IQ file through dsd:

sox -t raw -r 2048000 -es -b 16 -c 2 dstar.iq -t wav - | dsd -i -

  - Or use a real-time method with rtl_fm:

rtl_fm -f 445.525M -M fm -s 48000 - | dsd -i -

- Confirm Decode:
If D-STAR is active, you’ll see decoded text (callsigns, messages) and hear robotic voice output.

Would you like me to prep scripts for:

- Automating the frequency scan and heatmap generation?

- Running real-time D-STAR decode with fallback recording?

Let me know your preferred level of scripting and automation.

Steve:

Yes, let’s start with automating the frequency scan and heatmap generation.

ChatGPT:

Perfect. Let’s build a lightweight automation flow to scan 440–450 MHz, log the data, and generate a heatmap on your Raspberry Pi 4 with RTL-SDR.

Automation Script: Frequency Scan + Heatmap

Folder Setup

Choose a project folder to keep things clean:

mkdir -p ~/sdr-monitor/logs ~/sdr-monitor/output
cd ~/sdr-monitor

1. Install Prerequisites

A. Install RTL-SDR tools

sudo apt update
sudo apt install rtl-sdr python3-matplotlib python3-numpy sox git

B. Get the heatmap script

git clone https://github.com/keenerd/rtl-sdr.git
cp rtl-sdr/rtl_power/heatmap.py ~/sdr-monitor/

You now have rtl_power and heatmap.py.

2. Create scan.sh Script

Create a new file:

nano ~/sdr-monitor/scan.sh

Paste this script:

#!/bin/bash

# Variables
DATE=$(date +"%Y-%m-%d_%H-%M")
LOGFILE="logs/scan_$DATE.csv"
HEATMAP="output/heatmap_$DATE.png"

# Scan 440–450 MHz, 25 kHz steps, 15s integration, 15-minute scan
rtl_power -f 440M:450M:25k -i 15s -e 15m "$LOGFILE"

# Generate heatmap
python3 heatmap.py "$LOGFILE" "$HEATMAP"

echo "Scan complete: $LOGFILE"
echo "Heatmap saved to: $HEATMAP"

Save and exit (Ctrl+O, then Enter, then Ctrl+X).

Make it executable:

chmod +x ~/sdr-monitor/scan.sh

3. Test It

Run the scan:

cd ~/sdr-monitor
./scan.sh

After ~15 minutes, check ~/sdr-monitor/output for a PNG heatmap.

4. Optional: Schedule It (Crontab)

To scan every 6 hours:

crontab -e

Add:

0 */6 * * * /home/pi/sdr-monitor/scan.sh

Would you like:

- The heatmap timestamped on the image itself?

- A version that runs for 12–24 hours and outputs a compressed summary?

- Log parsing/summary script for most active frequencies?

Let’s keep iterating!